# P4P Presentation Planning

Reaction Wheel Microvibration, Imbalance & Torque Characterisation Platform — 8-minute presentation, split evenly between speakers.

# Overview

| Time | Speaker / Content |
| --- | --- |
| 0:00–4:00 | Brodie — motivation, objective, requirements (Slides 1–6) |
| 4:00–8:00 | Matthew — concept, design analysis, FEA, sensors, next steps (Slides 7–12) |

# Brodie's Slides (0:00 – 4:00)

### Slide 1 — Title / Introduction (30 sec)

Content:

- Project title, both names, department, supervisor name, date

Talking points:

- Introduce yourselves and the project name — no technical content yet

- One sentence on what the next 8 minutes will cover

- Keep energy up — this sets the tone for the whole talk

### Slide 2 — Problem Statement (1–1.5 min)

Content:

- One image of a satellite or RWA

- Three bullet points maximum: RWAs are the primary attitude control actuator on satellites; during operation they generate microvibrations that degrade payload performance; accurate characterisation of forces, torque, and imbalance is critical for mission success

- One quantified example — De Munter's 3 arcsecond pointing error figure

Talking points:

- Don't read the bullets — use the image to anchor the explanation

- Explain plainly what a reaction wheel does: it spins internally to torque the spacecraft without using fuel

- Even a well-balanced wheel isn't perfectly balanced — residual microvibrations leak straight through into the structure

- Use the De Munter number as the hook: a wheel exciting a 30 Hz structural mode caused 3 arcsec RMS pointing error — significant against a high-resolution imaging budget

- Land on: this is a measurement problem before it's a mitigation problem — you can't fix what you haven't characterised

### Slide 3 — Project Objective (30 sec)

Content:

- One sentence: “Design and build a unified measurement device for RWA microvibrations, imbalance, and torque”

- Two columns: Kistler image on left, your CAD on right

- Caption: “£10,000+ → £500 equivalent”

Talking points:

- State the objective as a single sentence — don't over-elaborate

- Let the £10k → £500 comparison land as the “so what” — commercial-grade capability at a fraction of the cost

- Quick pivot line into the state-of-the-art slide

### Slide 4 — State of the Art (30 sec)

Content:

- Small table or three icons: Kistler dynamometer / commercial balancing machines / existing literature

- One line each: what they do and why they're insufficient

- Gap statement: “No single device does all three simultaneously”

Talking points:

- Kistler tables measure force/torque well but don't give imbalance directly and cost £10k+

- Commercial balancing machines measure imbalance but not force/torque disturbance spectra

- Published models (e.g. Masterson) need per-wheel characterisation — no substitute for direct measurement

- Land clearly on the gap: nothing currently measures force, imbalance, and torque together in one integrated device

### Slide 5 — Measurement Requirements (30 sec)

Content:

- Small table: microvibrations / static imbalance / dynamic imbalance / torque

- Rocket Lab specs as targets (26 g·mm static, 800 g·mm² dynamic)

Talking points:

- Frame this as: before designing hardware, we needed numbers to design against

- Use Rocket Lab's flight qualification spec as a concrete, industry-grounded target — not an arbitrary number

- Close with a clean handoff: “that's the target — Matthew will take you through how we're hitting it”

### Slide 6 — Concept Overview — Six-Component Measurement & Ring Plate Concept (45 sec)

Content:

- Six-component measurement diagram (Fx, Fy, Fz, Tx, Ty, Tz)

- Ring plate concept sketch and why it was rejected

Talking points:

- Explain what “six-component” means physically: three forces + three torques fully characterise a wheel's disturbance signature

- Briefly mention the ring plate was the first concept explored — bearings on a rotating ring

- Rejected due to preload sensitivity and mechanical complexity — sets up why the stinger concept was chosen instead

- Keep this brief — it's a bridge into Matthew's decoupling problem slide, not a deep dive

# Matthew's Slides (4:00 – 8:00)

### Slide 7 — Decoupling Problem (30 sec)

Content:

- Simple diagram showing axial vs radial force directions

- Key point: radial and axial measurement must not interfere with each other

- “Stinger solves this — stiff in one direction, compliant in the other”

Talking points:

- Explain physically why cross-talk matters: if radial load bleeds into the axial channel, imbalance measurements get corrupted

- State the stinger principle in plain language — stiff in the direction you're measuring, floppy in the others

- Keep it brief, this is context for the concept comparison slide next

### Slide 8 — Two Concepts: Ball Bearing and Stinger (1 min)

Content:

- Side by side: ball bearing plate sketch on left, stinger CAD on right

- Two or three bullet points per concept

- Highlight why stinger was chosen — no preload, clean load path, six components

Talking points:

- Ball bearing plate needed preload to stay seated — an extra failure mode and extra complexity

- Stinger needs no pretensioning, gives a clean single load path, and only six components to manufacture and align

- Let the CAD image do the work — don't over-narrate the sketch

### Slide 9 — Stinger Design Analysis (1–1.5 min)

Content:

- One diagram showing stinger with d and L labelled

- Four check equations in a 2×2 grid — fn, stiffness ratio, buckling SF, yield SF

- Results table with checkmarks

Talking points:

- Say plainly: four checks per stinger, all four passed for the selected geometry (d = 2 mm, L = 50 mm, steel)

- Natural frequency > 500 Hz keeps the stinger's own resonance out of the measurement band (one-third rule: fn > 3× max measurement frequency)

- Stiffness ratio > 3000 keeps axial and radial channels decoupled — cross-axis error under 0.03%

- Buckling and yield margins are comfortable at this diameter — steel is well within capability

- Don't derive the equations live — just say “we check four things to make sure the stingers work” and walk through the results table

### Slide 10 — FEA Modal Analysis (45 sec)

Content:

- Mode 1 image (plate_mass screenshot)

- Frequency table — first 6 modes only

- One sentence on the limitation and planned fix

Talking points:

- Full 17-body assembly modal analysis in ANSYS — RWA represented as a 1.0 kg point mass at the rotor centre-of-mass height

- First six modes: two lateral translation (334 Hz, 397 Hz), torsion (696 Hz), axial bounce (893 Hz), two rocking modes (966 Hz, 999 Hz)

- Be upfront: Modes 1 and 2 sit below the 500 Hz target — that's support compliance in the radial stingers, not a fundamental flaw

- Fix already identified: stiffen the radial stinger supports (increased wall thickness), planned as the first Semester 2 design iteration

- “Mode 1 is below target due to support compliance — we've identified the fix for Semester 2”

### Slide 11 — Sensor Selection (30 sec)

Content:

- Simple table: 3× S-beam strain gauge (axial) / 3× PCB 208C01 ICP (radial)

- One line justification for each

- “Hybrid approach — each sensor type matched to its measurement role”

Talking points:

- Axial channel needs high-precision static/low-frequency load measurement — S-beam strain gauges suit that role

- Radial channel carries the higher-frequency microvibration content — ICP piezoelectric (PCB 208C01) is built for that bandwidth

- One line: each sensor type is matched to the physics of what it measures, not a single one-size-fits-all sensor

### Slide 12 — Next Steps (30 sec)

Content:

- Simple timeline or mini Gantt — not a bullet list

- Five items: sensor procurement → fabrication → assembly → testing → validation

- Dates aligned to your actual submission schedule

Talking points:

- Walk through the five-step path: sensor procurement → static FEA → detailed design → fabrication → validation

- Say the key dates out loud briefly — don't just point at the Gantt and move on

- End confidently: “by October we will have a working proof of concept”